Lesson Plan: Remote Sensing of Water Quality

Developed by Max Beal (mrbeal@wisc.edu) and Dr. Paul Block (paul.block@wisc.edu)

Department of Civil and Environmental Engineering, University of Wisconsin – Madison

Objective: To introduce students to the concept of remote sensing and its applications in monitoring water quality, specifically focusing on harmful algae. Students will engage in a coding activity to develop a statistical model correlating RGB values to chlorophyll-a levels in water samples. Students will then apply the statistical model to an image of Lake Mendota.

Duration: 1-2 class period (approximately 60-90 minutes)

Materials:

- PowerPoint presentation

- Camera (phone or computer)

- Computer and internet access

- Water quality samples with known chlorophyll-a levels

Procedure:

- PowerPoint Presentation (15 minutes)

  - Present the provided PowerPoint covering the following topics:

  - a. Harmful algae bloom: causes and negative impacts.

b. Definition and principles of remote sensing

c. Solar radiation and the ways it can interact with a medium (water)

  - Briefly introduce the activity (last slide)

- Coding Activity Explanation (5-10 minutes)

  - Explain the coding activity that students will be participating in using python and Google Colab, an open-source coding tool.

  - Describe the goal: to develop a statistical model that correlates RGB values of the four water sample images with their chlorophyll-a levels.

  - Students will need to create Google Colab accounts or access Google Colab from school google accounts.

- Coding Activity (25 minutes)

  - Divide students into pairs or small groups (2-3).

  - Provide computers and open the code notebook (https://tinyurl.com/MendotaRS). Make a copy of the notebook to their personal account (File > Save a Copy in Drive).

  - Students create water quality samples in a 0.5 liter cup of water by adding chlorophyll following the table below:

| Volume chlorophyll to add | # 3ml droppers | Chlorophyll-a (ug/L) |
| --- | --- | --- |
| 1.5ml | 0.5 | 0.15 |
| 3ml | 1.0 | 0.17 |
| 15ml | 5.0 | 1.08 |
| 25ml | 8.3 | 2.85 |

Example Photos:

  - Instruct students to take pictures of each water quality samples using either a phone or computer camera. Try to make the background of each photo similar. Using a sheet of white paper with the concentration written on the background may be helpful.

  - Guide students in uploading photos and running the provided code.

- Data Analysis and Conclusion (10 minutes)

  - Have Students complete part 1 (reading images and calculating reflectance) and discuss

    - Touch base and make sure everyone could get to this point. Compare results.

    - Discuss questions and how each group’s results differ

  - Have students complete part 2 and 3. analyze the statistical model and interpret the results.

    - Talk about which models performed the best by R2 and which models make the most sense when compared to the original hypotheses.

    - Discuss how the model translated to the image of Mendota and what limitations might exist for the model.

  - Encourage students to discuss the significance of their findings and any limitations or improvements that could be made.

  - Conclude the lesson by summarizing the usefulness and potential limitations of remote sensing in assessing water quality and its potential for environmental monitoring.

Homework:

- Questions are provided and can be answered in the coding document. Additional example questions are provided for discussion after if needed.